Aircraft washing truck

Specifications:

- Basic vehicle: Volvo FL 6.14 or similar with PTO

- Polyester water tank:	8 000 litres

- Poly urethane soap tank:	   200 litres

- Hydraulic system build with Rexroth components

- Hydraulic driven water pump and auxiliary systems simultaneously.

- Bypass system with heat exchanger to avoid overheating of water pump and hydraulic system.

- 5 hydraulic hose reels with 80 metres of hose each.

- Oversized 7-stage centrifugal water pump with a capacity of 360 litres/minute at 18.2 bar, thanks to this configuration there will be hardly any pressure difference when using one or 5 hoses.

- Water activated adjustable soap injector

- Work area lightning by 2 adjustable high performance HQI lights

- 220 Volts supply by a reinforced 24 Volts alternator, combined with a 220 Volts sinusoid inverter of 2 500 Watt.

- Stair to the stainless steel hatch for refilling and cleaning of the tank.

- CE approved

- 2 year warranty on parts

## Options

- Extra valves for outboard hydraulic appliances

- Hydraulic soap dispenser

- Construction to customer specification